# Connecting Multiple VTI LED Posters

# Connecting Hardware

Below familiarize yourself with your LED Poster’s basic input and output ports.

## Multi-Screen Display

To connect two or more LED Posters, you simply need to connect the “HDMI OUT” of Poster1 to the “HDMI IN” of Poster2. (see right)

| Poster1: HDMI OUT | Poster2: HDMI IN |
| --- | --- |
| ## Seamless Video Wall To fit LED Posters together for a seamless Video Wall you only need to: - Remove the magnetic sidebars (see right) between two posters. - Station LED Posters next to each other (see below). |  |

- Remove the modules between two posters, which cover the *sidelock of the Poster cabinet

|  | Use magnet included in LED Poster kit. |
| --- | --- |

*Side locks (pictured below)

- Wrench the side lock and screws as shown in the attached video.

- Connect the Poster1 HDMI OUT to the Poster2 HDMI IN.

## Display Content - Viplex Handy

- Connect to Screen

  - Click Set Network to access the WLAN connection page of the mobile phone.

  - Connect the WiFi AP of Taurus series products. (The default SSID is the last 8 digits of AP+SN, and the password is 12345678.)

- Run ViPlex Handy Mobile App

| Android | iOS |
| --- | --- |
| Download Link | Download Link |

- xs

- Enter Device Management

- Select the Multi-Screen Mosaic

  - Steps (shown in Red Numbering above):

    - 1) Setting of splicing direction; (If the software is connected to the leftmost poster, choose the direction from left to right (facing the screen))

    - 2) Splicing method: according to the width or according to the number, we choose "quantity"

    - 3) The number of splicing should be consistent with the number of screens.

    - 4) Press “Done”.